SENSOR EVALUATION & DEPLOYMENT DOCUMENT

AI System to Detect Intentional Railway Track Tampering

SECTION 1: RECOMMENDED (CORE) SENSORS

(Continuous Monitoring – Always ON)

MEMS ACCELEROMETER (VIBRATION SENSOR)

1. Installation

- Mounted on rail web (side of rail) or sleeper

- Fixed using industrial clamp or magnetic mount

- No drilling → no structural weakening

- Weatherproof enclosure (IP65)

2. How It Takes Readings

- Measures acceleration in X, Y, Z axes

- Sampling rate: 800–3200 Hz

- Outputs continuous time-series vibration data

3. How It Detects Anomaly

- Establishes baseline vibration profile

- Uses:

  - Frequency analysis (FFT)

  - Pattern recognition (ML)

- Detects:

  - Sudden spikes

  - Tool-induced vibration frequencies

  - Repetitive hammering patterns

4. Why It Is Better

- Detects almost all physical tampering

- Works day/night, rain/fog

- Already used in rail & bridge monitoring

5. What It Solves

Hammering
Grinding / cutting
Track shaking
Fastener loosening
Forced rail movement

6. What It Can’t Solve

Cannot visually identify intruder
Cannot detect silent slow tampering alone

7. Minimal Cost (Sensor + Installation)

- Sensor: ₹300 – ₹800

- Mount & casing: ₹500

- Total: ₹800 – ₹1,300 per unit

8. How It Sends Readings

- Wired to edge controller (ESP32 / STM32)

- Data sent via:

  - LoRaWAN / LTE / Rail fiber (gateway)

9. Power Supply

- Solar panel + Li-ion battery

- Or trackside power

- Very low power (<10 mW)

MEMS MICROPHONE (ACOUSTIC SENSOR)

1. Installation

- Mounted near rail but vibration isolated

- Enclosed in wind-shielded casing

2. How It Takes Readings

- Samples audio at 16–44 kHz

- Captures environmental sound waves

3. How It Detects Anomaly

- FFT extracts frequency bands

- ML classifies:

  - Cutting noise

  - Grinder sound

  - Hammer rhythm

4. Why It Is Better

- Detects tool sounds even with weak vibration

- Excellent confirmation layer

5. What It Solves

Hacksaw cutting
Grinder usage
Repetitive metal strikes

6. What It Can’t Solve

Wind & rain noise alone
Cannot detect silent misalignment

7. Minimal Cost

- Sensor: ₹300 – ₹600

- Casing & mount: ₹400

- Total: ₹700 – ₹1,000

8. Data Transmission

- Audio features (not raw audio) sent

- Reduces bandwidth usage

9. Power

- Same power unit as accelerometer

- Duty-cycled for efficiency

TILT / IMU SENSOR

1. Installation

- Mounted rigidly on rail web or sleeper

- Fixed orientation

2. Readings

- Measures roll & pitch angles

- Continuous angle tracking

3. Anomaly Detection

- Baseline alignment stored

- Sudden angle change → alert

- Gradual drift → maintenance flag

4. Why It Is Better

- Detects structural shift

- Excellent for post-tampering impact

5. Solves

Rail misalignment
Track bending
Support removal

6. Can’t Solve

Cannot detect who caused it
No detection of tools directly

7. Cost

- Sensor: ₹300 – ₹700

- Installation: ₹300

- Total: ₹600 – ₹1,000

8. Data Transfer

- Angle data sent periodically

- Event-based transmission

9. Power

- Ultra-low power

- Shares common power unit

MAGNETIC FIELD SENSOR

1. Installation

- Installed near:

  - Fish plates

  - Bolts

  - Joints

2. Readings

- Measures local magnetic field strength

3. Anomaly Detection

- Sudden magnetic drop → metal removed

- Sudden spike → tool proximity

4. Why It Is Better

- Detects metal removal

- Adds redundancy

5. Solves

Bolt removal
Joint tampering
Rail cutting events

6. Can’t Solve

Affected by passing trains alone
Needs fusion with other sensors

7. Cost

- Sensor: ₹200 – ₹400

- Installation: ₹300

- Total: ₹500 – ₹700

8. Data Transfer

- Low-frequency magnetic data

- Event-based alerts

9. Power

- Extremely low power

- Battery/solar powered

SECTION 2: OPTIONAL / ADVANCED SENSORS

(Triggered / Verification Only)

CAMERA (TRIGGER-BASED)

Solves

Visual evidence
Intruder confirmation

Can’t Solve

Continuous monitoring everywhere

Cost

₹2,000 – ₹5,000 per unit

Triggered only when anomaly detected.

PIR SENSOR

Role

- Human presence trigger only

Limitation

- Animals & heat false positives

Cost

₹200 – ₹400

ULTRASONIC / LiDAR

Role

- Object detection on track

Limitation

- Rain, fog, cost

Cost

- Ultrasonic: ₹300 – ₹500

- LiDAR: ₹6,000+

STRAIN GAUGE (PILOT ONLY)

Role

- Stress & deformation detection

Limitation

- Installation complexity

- Long-term stability

Cost

₹1,500 – ₹3,000

FINAL SYSTEM COST ESTIMATION (PER LOCATION)

MINIMAL DEPLOYMENT (CORE ONLY)

| Component | Cost |
| --- | --- |
| Accelerometer | ₹1,000 |
| Microphone | ₹800 |
| IMU | ₹800 |
| Magnetic sensor | ₹600 |
| Edge controller + power | ₹2,000 |
| Total | ₹5,200 – ₹6,000 |

FULL DEPLOYMENT (WITH OPTIONAL)

| Add-ons | Cost |
| --- | --- |
| Camera | ₹3,000 |
| PIR | ₹300 |
| Ultrasonic | ₹400 |
| Total Add-on | ₹3,500 – ₹4,000 |

FINAL TOTAL COST

- Minimum: ₹5,200 per node

- Maximum: ₹10,000 per node

(Scalable, modular, realistic)