1. The Complete Tech Stack ()

| Component | Technology Selected | Why It Was Chosen |
| --- | --- | --- |
| Microcontroller (Edge) | ESP32 (or STM32) | Dual-core, built-in Wi-Fi/BT, supports Deep Sleep, cheap ($5). |
| Sensors | ADXL345 (Accel), HMC5883L (Mag), INMP441 (Mic) | Digital interfaces (I2C/I2S) for noise-free data; low power. |
| Communication Protocol | MQTT (over LoRaWAN or Wi-Fi) | Lightweight, reliable, designed for unstable IoT networks. |
| Backend API | Python (FastAPI) | High performance (Async), native AI integration. |
| Message Broker | Mosquitto (or HiveMQ) | The "Post Office" that routes messages between sensors and server. |
| Real-Time DB | InfluxDB | Stores high-speed time-series sensor data. |
| Operational DB | PostgreSQL | Stores user data, sensor locations, and alert logs. |
| Frontend | React.js + Tailwind CSS | Fast, responsive dashboard for officials. |
| Live Updates | Socket.io (WebSockets) | Pushes alerts to the screen instantly (no refresh needed). |

2. How Communication Works (From Track to Screen)

The system does not use a direct cable. It uses a Publish-Subscribe (Pub/Sub) model via MQTT. This acts like a "Twitter feed" for machines.

The Flow:

- The "Publisher" (The Sensor Node):

  - The ESP32 reads the vibration data.

  - It packages this data into a JSON packet: {"id": "Node_45", "vib": 2.5, "status": "ALERT"}.

  - It publishes this message to a specific "topic" (channel) called: railway/track_45/data.

- The "Broker" ( The Middleman):

  - The Mosquitto MQTT Broker (living in the Cloud/Server) receives this message.

  - Its job is simply to hold the message and pass it to anyone listening. It doesn't process it.

- The "Subscriber" (The Backend API):

  - Your FastAPI Backend is constantly listening (subscribed) to railway/+/data.

  - As soon as the Broker gets the message, it instantly pushes it to the Backend.

- The "Final Mile" (To the Dashboard):

  - The Backend processes the data (runs AI check).

  - If it's an emergency, it uses Socket.io to push a notification to the React Frontend.

  - The Official's screen flashes RED immediately.

3. How Synchronization Works (Avoiding Discrepancy)

One of the biggest risks in IoT is "Data Drift" or "Packet Loss." (e.g., The sensor detects a cut at 10:00 AM, but the server receives it at 10:05 AM due to bad network).

Here is how we solve it:

A. Time Synchronization (The "Heartbeat" Clock)

- Problem: The ESP32's internal clock is cheap and drifts (loses seconds) over time.

- Solution: NTP (Network Time Protocol).

  - Every 6 hours, the ESP32 wakes up and asks the Server: "What time is it?"

  - It resets its internal clock to match the Server's atomic clock exactly.

  - Result: Every single data packet is stamped with the exact real-world time (timestamp: 1735987200) at the moment of creation, not the moment of arrival. Even if the internet cuts out for an hour, when the data finally arrives, the server knows exactly when the event happened.

B. Data Buffering (The "Black Box" Recorder)

- Problem: The train passes through a tunnel or remote area where the network (LoRa/4G) drops.

- Solution: FIFO Buffer (First-In, First-Out).

  - The ESP32 has a small internal memory (Flash/SD Card).

  - If the network fails, it saves the sensor readings locally.

  - As soon as the network returns, it "burst transmits" all the saved data in order.

  - Result: Zero data gaps in your graphs.

C. QoS Levels (The "Read Receipt")

- Problem: The sensor sends an ALERT, but the message gets lost in the air.

- Solution: We set MQTT to QoS Level 1 (Quality of Service).

  - QoS 0: Fire and forget (Unreliable).

  - QoS 1 (We use this): The Sensor keeps sending the "ALERT" message again and again until the Server sends back a PUBACK (Publish Acknowledged) receipt.

  - Result: Critical alerts are guaranteed to arrive.

D. Duplicate Filtering (The De-Duplicator)

- Problem: Because of QoS 1, the server might receive the same "ALERT" message twice.

- Solution: Idempotency Keys.

  - Every message has a unique ID (msg_id: 887766).

  - The Backend checks: "Have I seen ID 887766 before?"

  - If Yes -> Ignore. If No -> Process.

  - Result: No double-counting of accidents.

Summary of Sync Strategy

| Feature | Mechanism | Benefit |
| --- | --- | --- |
| Exact Timing | NTP Sync | Timestamp reflects the event, not the upload. |
| Network Failure | Flash Buffering | No data is lost during internet outages. |
| Message Loss | MQTT QoS 1 | System retries until delivery is confirmed. |
| Duplicate Data | Unique Message IDs | Prevents false double-alerts. |